# Written Explanations

Student ID → base network conversion

Our group's base network was derived from the student IDs of the first two members. Member 1's ID (23101162) contributes the last two digits, 62, and Member 2's ID (23101214) contributes the last two digits, 14. These two numbers form the first two octets of our address space, giving a base network of 62.14.0.0/16. Every subnet used across all six units and their interconnecting links was carved out of this single /16 block using VLSM, sized according to each unit's actual host requirement rather than a fixed uniform size.

# DHCP design

Our network uses three different DHCP delivery methods depending on the unit's requirement. SCL and MDU each run a dedicated, server-based DHCP service (on SCL-DHCP and MDU-DHCP respectively), directly attached to their own LAN. RMU and DBG instead use router-based DHCP, where the pool is configured directly on the router itself using ip dhcp pool, with the starting address of each pool offset according to a specific group member's student ID digits — RMU's pool begins at 62.14.4.62 (Member 1's digits) and DBG's begins at 62.14.6.14 (Member 2's digits). GVU has no DHCP server of its own; instead, its router is configured with ip helper-address pointing at SCL-DHCP's IP (62.14.0.4), relaying DHCP requests from GVU's PCs across the network to be served by a second pool configured on SCL-DHCP specifically for GVU's subnet. SCL's own router also carries a helper-address configuration for consistency, even though its DHCP server is local. ASN is the only unit with no DHCP at all — every device there, including its two PCs, uses static addressing, per the requirement that ASN remain fully statically configured.

Packet Tracer’s protected serverPool could not be removed. Therefore, the SCL /22 DHCP address space was divided into two valid, non-overlapping ranges. SCL_POOL provides 520 addresses from 62.14.0.7 through 62.14.2.14, while serverPool provides the remaining 496 addresses from 62.14.2.15 through 62.14.3.254. Both ranges use gateway 62.14.0.1, DNS server 62.14.0.2, and subnet mask 255.255.252.0.

# Static routing design

| Five required static-routing types were implemented. SCL uses a next-hop static route to reach the ASN LAN |  |  |  |  |
| --- | --- | --- | --- | --- |
| through GVU at 62.14.8.210. ASN uses exit-interface static routes through GigabitEthernet0/1 for the other unit |  |  |  |  |
| LANs and also uses a default static route through the same interface. RMU uses a recursive static route to reach the |  |  |  |  |
| MDU LAN through MDU’s backbone address, 62.14.8.195. DBG uses a floating static route to the SCL LAN |  |  |  |  |
| through MDU at 62.14.8.222 with administrative distance 123. Because the normal RIP route has a distance of 120, |  |  |  |  |
| it remains preferred; when the primary DBG–SCL link was disabled, the floating route became active as designed. |  |  |  |  |

# Dynamic routing design

RIPv2 is enabled on SCL, RMU, DBG, MDU, and GVU with automatic summarization disabled. Their LAN interfaces are configured as passive interfaces so the routers advertise the LAN networks without sending unnecessary RIP updates to end devices. ASN does not participate in dynamic routing, as required. GVU has a static route to the ASN LAN, 62.14.8.128/26, through 62.14.8.217 and redistributes that static route into RIP. This allows the other five routers to learn how to reach ASN while ASN returns traffic using its static and default routes.

# Which router uses both static and dynamic routing

GVU uses both static and dynamic routing. It runs RIPv2 to exchange routes with SCL, MDU, and the rest of the routed network, while using a static route to reach the ASN LAN through ASN’s transit address, 62.14.8.217. The redistribute static command under GVU’s RIP configuration advertises the ASN LAN to the other RIP routers.